ELECTRICAL CONTROL OF INDUSTRIAL MECHANISMS · PLC-200 TRAINER · S7-200 CPU 224

LAB WORK REPORT

One report per group. Attach the program listing printed from STEP 7-Micro/WIN. The “Initials” boxes are signed by the instructor during the session. Grey italic text is an example: replace it.

| LAB TITLE |  | LAB NO. 0 · 1 · 2 · 3 · 4 · 5 · 6 |
| --- | --- | --- |
| GROUP | BENCH NO. | SESSION DATE |
| INSTRUCTOR |  | CPU AND FIRMWARE READ |

| PREPARATION | PROGRAM | BENCH | REPORT | TOTAL |
| --- | --- | --- | --- | --- |
| …… / 3 | …… / 6 | …… / 6 | …… / 5 | …… / 20 |

0  THE GROUP

| FULL NAME | STUDENT ID | ROLE AT THE BENCH | SIGNATURE |
| --- | --- | --- | --- |
|  |  | Programmer / Tester / Reporter |  |
|  |  | Programmer / Tester / Reporter |  |
|  |  | Programmer / Tester / Reporter |  |

Roles: programmer, tester (operates the bench and fills in the log), reporter. Swap roles from one lab to the next.

1  PREPARATION     ( / 3 )

Done before the session. For each preparation question: the answer, the working, the value and its unit.

| QUESTION | ANSWER OR WORKING | VALUE | UNIT |
| --- | --- | --- | --- |
| P1 | Preset of T37 for 5 s, 100 ms base: 5 / 0.1 | 50 | × 100 ms |

2  FUNCTIONAL ANALYSIS

2.1 INPUT / OUTPUT AND SYMBOL TABLE

| ADDRESS | SYMBOL | BENCH DEVICE | NO / NC | COMMENT |
| --- | --- | --- | --- | --- |
| I0.1 | STOP_NC | STOP switch | NC | 1 at rest, 0 when pressed or wire broken |

A stop, emergency-stop or safety limit device is wired NC: at rest the input reads 1, and a broken wire makes it 0.

2.2 GRAFCET

GRAFCET Draw or paste the GRAFCET here (numbered steps, transition conditions written with the symbols, actions to the right of the steps).

☐  One initial step (double square)

☐  Every transition has a condition

☐  Timers are written t/Xi/duration

☐  Stored actions have their reset

GRAFCET APPROVED BEFORE PROGRAMMING · INITIALS

2.3 INTERNAL RESOURCES USED

| RESOURCE | ADDRESS | PURPOSE |
| --- | --- | --- |
| Timer | T37 | Duration of step 1 (5 s) |

3  PROGRAM     ( / 6 )

Attach the ladder listing and the symbol table printed from Micro/WIN. Here, the map of the program: one network per row.

| NETWORK | NETWORK TITLE | STEP OR FUNCTION |
| --- | --- | --- |
| 1 | Initialisation on the first scan (SM0.1) | Step 0, safety, display… |

☐  Every network has a title and a comment

☐  Symbols are used everywhere, no bare addresses

☐  Each output is written only once

☐  Safety conditions (stop, limit switches) act outside the steps

4  TEST LOG

Write the expected result before the test, then what you observe on the bench.

| NO. | ACTION OR CONDITION | EXPECTED | OBSERVED | OK? | INITIALS |
| --- | --- | --- | --- | --- | --- |
| 1 | Press START, STOP at rest | Q0.0 = 1 and stays at 1 |  |  |  |

SIGNED CHECKPOINTS

| CHECKPOINT | EXPLAINED BY | TIME | INITIALS |
| --- | --- | --- | --- |
| PC – CPU communication |  | 10:40 |  |

The instructor signs after a group member who did not program the network explains it.

5  FAULT INJECTION     ( bench / 6 )

Every fault listed in the lab sheet is really provoked. A fault must stop the machine, never start it.

| FAULT PROVOKED | HOW | EXPECTED SAFE STATE | OBSERVED | SAFE? | FIX | INITIALS |
| --- | --- | --- | --- | --- | --- | --- |
| STOP wire broken | I0.1 to OFF, motor running | Q0.0 = 0, no restart on its own |  |  |  |  |

6  DISCUSSION     ( / 5 )

ANSWERS TO THE LAB QUESTIONS

DIFFICULTIES MET AND HOW YOU SOLVED THEM

POSSIBLE IMPROVEMENTS

GROUP DECLARATION

☐  We declare that this report and the attached program are our group’s own work. Any outside help (classmates, documents, AI tools) is listed below.

HELP AND SOURCES USED

| SIGNATURE, MEMBER 1 | SIGNATURE, MEMBER 2 | SIGNATURE, MEMBER 3 |
| --- | --- | --- |

✓  MARKING GRID     ( instructor )

| PART | WHAT IS ASSESSED | MARK |
| --- | --- | --- |
| Preparation | I/O table and GRAFCET correct before the session, preparation calculations | …… / 3 |
| Program | Behaviour; symbols, SCR steps, comments | …… / 6 |
| Bench demonstration | Functions shown; every fault-injection test ends in a safe state | …… / 6 |
| Report | Complete, clear, tables filled in and signed, answers justified | …… / 5 |
| Total |  | …… / 20 |

INSTRUCTOR’S REMARKS